WB-0BC-2NT
2.4Ghz 200m Elevator Wireless Bridge

The dual wireless bridge can be applied in industries, such as wireless video security, railway, transportation, power and other industries, wireless video/data transmission, wireless coverage, carrier wireless backbone network construction, Wi-Fi wireless coverage, rural information construction, and wireless network coverage. The long-distance transmission of the device can guarantee multi-channel HD video transmission.

- Up to 200 m wireless transmission distance

- 802.11n wireless bridge

- Ideal Solution for Elevator Connection

- Plug and Play

| - Specification |
| --- |
| Wireless Parameter |  |
| Max. Transmission Rate of Air Interface | ≤ 72 Mbps |
| Wireless Standard | IEEE 802.11b/g/n |
| Working Frequency Band | 2400 MHz ~ 2483.5 MHz |
| Antenna Gain | 7 dBi |
| Antenna Angle | (H) Beamwidth [3 dB] 50° (V) Beamwidth [3 dB] 50° |
| Transmitting Power | 16 dBm @11b 1Mbps (Note: Transmitting power may vary with local regulations.) |
| Receiving Sensitivity | -85±2dBm @HT20 MCS0 |
| Throughput Rate | PTP: 30 Mbps/0.2 KM(HT20) |
| Hardware Parameter |  |
| Network Interface | 1 × 10/100 Mbps RJ45 ports |
| LED Indicator | Power indicator,LAN indicator,Signal intensity indicator |
| Reset | Hardware resetting button |
| Power Supply | 12-24 VDC Passive PoE |
| Software Function |  |
| Channel Width | 10/20 MHz |
| Channel Selection | Auto/manual selection |
| Network Protocol | NTP (network time synchronization),SADP (auto search for IP address),SSH (debugging) |
| Security Mode | WPA2-PSK |
| Security Mechanism | Wireless network name hidden |
| Management Method | Web |
| Upgrade | Web |
| System Log | Syslog |
| Application Function | Support adaptive frequency hopping. Support fault self-healed. |
| General |  |
| Product Dimensions (W × H × D) | 80 mm × 41.94 mm × 136 mm (3.15'' × 1.65'' × 5.35'') |
| Package Dimensions (W × H × D) | 256 mm × 231 mm × 86 mm (10.07" × 9.09" × 3.38") |
| Net Weight | 0.12 kg (0.26 lb) per device |
| Gross Weight | 0.7 kg (1.54 lb) |
| Packing List | Wireless Bridge × 2, Power Adapter (12 VDC, 1 A) × 2, 10/100 Mbps PoE Injector × 2, Binding Tape × 4, Quick Start Guide ×1, Regulatory Compliance and Safety Information × 1 |
| Packing Method | Packing in pairs |
| Working Temperature | -30 °C to 70 °C (-22 °F to 158 °F) |
| Working Humidity | 5% to 95% (no condensation) |
| Storage Temperature | -40 °C to 80 °C (-40 °F to 176 °F) |
| Storage Humidity | 5% to 95% (no condensation) |

| Installation Mode | Pole Mounting |
| --- | --- |
| Ingress Protection | IP41 |
| Power Consumption | 1 W for typical (1-ch 2MP IPC), Max. 2 W |
| Approval |  |
| EMC | CE-EMC (EN 55032: 2015+A11: 2020, EN IEC 61000-3-2: 2019, EN 61000-3-3: 2013+A1: 2019, EN 50130-4: 2011+A1: 2014, EN 55035: 2017+A11: 2020) |
| Safety | CB (AMD1:2009, AMD2:2013, IEC 62368-1: 2014 (Second Edition)), CE-LVD (EN 62368-1: 2014+A11: 2017) |
| Chemistry | CE-RoHS (2011/65/EU),WEEE (2012/19/EU),Reach (Regulation (EC) No.1907/2006) |
| Radio | EN 301 893,EN 300 440,EN 62311,EN 301 489-3,EN 301 489-17 |

- Typical Application

- Physical Interface

| No. | Interface/Button | Description |
| --- | --- | --- |
| 1 | Reset | Long press for 4 seconds to restore to factory settings. |
| 2 | PoE/LAN | The 10/100 Mbps adaptive RJ45 port, also can be used as a PoE port. |

- Available Model WB-0BC-2NT

- Dimension